| To : | Doc. No. | SECYYMM-XX-X |
| --- | --- | --- |
| C.C. : | Country | ALL |
| ## From: | Tel: 82-31-279- Fax: E-mail: @samsung.com |  |
| Subject : SMT-i5230 SW Release |  |  |

Overview :

- SMT-i5230 S/W : V1.30 (2013.03.11)

## New Version Changes

’V01.30(2013.03.11) :  5 Dec, 201

* New feature

1. Multicasting packet processing scheme on SMT handset has been improved.

When IP multicasting heavily occurs in the network, it will affect the packet loss or slow operation on the SMT handset.

So, SMT handset has been improved not to respond to a multicast packet coming into the port of SMT handset(especially 8384) except in the following 3 ports.

- 8384 : OS communicator

- 9000,9001 : RTP, RTCP

* Fixed problem

1. When a SMT handset’s DHCP address is updated, the phone has not notified it to OfficeServ system. It has been fixed.

’V01.29(2012.12.05) :  5 Dec, 2013

* New feature

1. Polish has been added as new language option.

2. The SMT handset type migration (through SW upgrade) between SCME and OSPP has been restricted. (STA only)

3. The size of the call-log history has been expanded from 16 to 32.

* Fixed problem
1. A character length of IP phone ID/Password extended to 32 bytes.
   - The ID and password length of SCME phone is max. 32 characters and that of OSPP phone is max 4 characters. It doesn’t allow user to insert an id and password prior to the phone type exchange from OSPP to SCME. It has been fixed.

2. Default value changed
* Menu-> 7.Settings-> Application Set.->5.Headset Key

- After the factory reset, [Headset Key] menu’s default value changes from ‘Not Use’ to 'Use’.

And when the headset key has been pressed and the headset menu has set to ‘Not use’, this status notification will be displayed on a pop-up window for reminding a user of it.

3. The SMDR message had been overlapped on the date information on the screen due to a text alignment problem. It has been fixed.

’V01.27(2012.03.08) :  8 Mar, 2012

* New feature

- Additional items for AFT(Automatic funtion test) in the production phase, has been implemented to this S/W version. 
Following items have been added.
- Memory Test / Boot mode(DHCP) Test / Save the test result

’V01.26(2012.02.04) :  4 February, 2012

* Fixed problem

- Paging call operation didn’t work correctly.
 IGMP JOIN Message from SMT-i5230 didn’t send when the phone received a multicast paging call.

- Several Italian strings have been replaced to correct ones.

’V01.25(2011.12.20) :  20 December, 2011

* New feature

Interoperatibilty with OfficeServ V4.60

For more details, refer to System Administration Guide for new features.

1. SRTP function is added.

 SRTP is enabled or disabled by the system DB.

- PnP function is enhanced.(PreMAC, Auto PnP, Normal Login Mode)

PnP operation is configured by the system DB. When the IP phone is in the PnP mode, it operates differently according to system configuration.

- System phonebook feature is added.

When the system phonebook is added to system DB, IP phone downloads this phonebook and uses it. When the system phonebook is downloaded to IP phone, system phonebook menu is displayed in the phone’s phonebook menu.

- Multicast paging is supported.

 When the page group is called, multicast RTP is sent to each IP phone.

- Auto upgrade function is added.

* Fixed problem

- When the SMT-i5230 operates with the MCP(V4.46, not MP), it cannot have a normal conversation.

 Bug Fixed. Because of inconsistency with the MCP, SMT-i5230 has a call using SRTP.

- During ringback heard on the set, campon tone is generated at the same time becaue of call waiting from another phone.In the SMT phones, ringback tone is stopped when a campon tone is generated.

- Modified that ringback tone is not stopped when a campon tone is generated.

- When the IP phone is in the PnP mode and user enter wrong ID or password, IP phone has to be rebooted to enter ID or password again.

 When an error response is received, IP phone returns to login window to enter ID or password again.

- When the IP phone uses 8 character password, IP phone cannot register to system. (User password is set to use 8 characters on the system DB). Bug Fixed.

6. Oneway is occurred very occasionally when starting a normal call.

 Bug fixed.

- Sometimes no speech situation is occurred when starting a normal call and IP phone has to be rebooted for solving this problem.

 Bug fixed.

’V01.15(2011.10.05) :  5 October, 2011

* Fixed problems

1. Slow operation issue in engineer mode has been fixed.

When SMT-i5230 phones are installed at the customer site at very first time, the phones boot up in PnP mode but cannot get an IP address from DHCP server because most sites do not set DHCP severs. Therefore, installers should go into the engineer mode in order to set configuration of IP phones such as IP address, Server IP address and so on.

When installers try to go into engineer mode, the slow operation occurs.

This issue only occurs in V1.14 (2011.05.23) and when phones produced with V4.14 package.

2. DHCP issue has been fixed.

Some DHCP servers assign the same IP address for several IP phones because these assign IP address referring to Transaction ID in DHCP Discover message which sent by IP phones. In order to avoid it, we change the way to generate a unique Transaction ID like below.

|  | Current | New |
| --- | --- | --- |
| Random number seed | Time value | MAC Address |

’V01.14(2011.05.23) :  23 May, 2011

* Fixed problems

- Qnet [P110215001] : Delayed speech error has been fixed.
Approx 700ms speech delay happened after answering a call and now this delay has been shorten to 200~300ms.

’V01.13(2011.04.12) :  12 April, 2011

* Fixed problems

1. Key tone was changed(Menu -> 4. Sound -> 3. Keytone( DTMF  BEEP))

2. Menu display Error in Russia Language Mode was fixed.

3. Key tone was changed(Menu -> 4. Sound -> 3. Keytone( DTMF  BEEP))

4. Initialize all led and icons at logout state.(Extension login mode)

5. OSC login password has been unified same as Security password of IP Phones.

| Phone | Version/Date | Version/Date |
| --- | --- | --- |
| SMT-i3100 | ~ V01.33(2010.07.22) | V1.41 (2011.04.08) ~ |
| SMT-i5210 | ~ V01.11 (2010.07.22) | V1.17 (2011.04.08) ~ |
| SMT-i5230 | ~ V01.11 (2010.07.21) | V1.13 (2011.04.12) ~ |
| Password for OSC Login on IP Phone | System Login password will be used from above versions of package. | Security password (which is used for Phone Menu lock) has been used. |
| SMT-i5220 | Security password (which is used for Phone Menu lock) has always been used for OSC login on IP phones. |  |
| SMT-i5243 |  |  |

’V01.11(2010.07.21) :  21 July, 2010

* Fixed problems

1. Special code has beed added in forward setting mode.

Below code is possible to input.

F,f  (0x0c): Flash Code

P ,p (0x0d): Pause Code

C ,c (0x0e): Change PTMFC CODE, Dial way

[, or ] (0x0f): OCC Code, Encryption

2. All of the Mutilingual String have been included with reference to your requests.

(Not included in case of Softmenu list strings yet. That depends on the system)

3. Headset volume issue has been fixed. (Headset volume falls back to previous call's volume level.)

4. Warning message about 'Busy/NoAns forward' has been modified to clarify the meaning.

5. VLAN setting error in the Easyinstall menu of Engineer mode has been fixed.

6. Phonebook setting error in adding entry has been fixed.

’V01.07(2010.04.05) :  5 Apr, 2010

* Fixed problems

1. Soft key lock
When the phone receives a new message in enbloc mode and the new message  window is popped up, soft keys are locked up if any button of dial pad is pressed.

’V01.07(2010.04.01) :  1 Apr, 2010

* Fixed problems

- Changed Default  value. (Menu-> 7.Settings-> Application Set.->4.Short Message)
After  Factory Reset,  Changed [ Msg Contents View] menu option(OFF  ON)

## ’V01.06(2010.03.26) :  26 Mar, 2010

* Fixed problems

- If you are on a call and you scroll thru the options either on the Func or AOM menus speech is cutoff i5230 V1.03, OS7400 V4.46a 10.02.26

- Added PageUp and Page Down soft men in AOM or Func. Menu.

- when reviewing a short message the time format is in 12Hr, I think the time format should follow what you have set on your phone. Send short message to ext 3201, review the message on ext 3201 time format is in 12Hr when phone is set to 24Hr. All SMT-I phones.

- Added Headset Key menu.(Menu -> 7 Settings -> 1 Application Setting -> 7 Headset Key)

- i5230 V1.03 date display changed from Western to Oriental when headset button is toggled on and off.

’V01.03(2010.03.03) :  03 Mar, 2010

* Fixed problems

1. Changed Phone Menu

- Samsung UI Guide format is adapted in this version.

2. In Call sate, AOM Menu is added in Soft Menu.

- in old version, AOM is displayed in idle state,

but this version AOM is display call state(ringing, ring back, conv state).

3. STA field issue is fixed.

- Need improvement on AOM key function

- MMC 115 (PGMMSG) When using the volume key to scroll through the 2 digit program messages, the display will change from a 2 digits message (00) to a 3 digit message (010.

- MMC 115 (PGMMSG) The ) parenthesis is missing at the far right of the display.

- During the call state, the phone do not have a soft button option to allow access to the AOM. Customers will need this so that they can quickly  and often transfer calls to other internal stations.

- When set to Enblock and LCR, dial 9-2143334444, before hitting the call or send there are no soft button options to backspace or clear the entry. The current IP phone have soft keys to clear, the 5243 has a cancel soft key to delete digits.  The 5230 has a Del soft key to delete digits.  In consistent.  Make the same for all phones.

-. Call LED has wrong flash rate during DND state

- Need to have backlight when user uses the phone even backlight is turned off at the schedule time

- Service key and Menu for service key is showing different options on the 5210 phone.  Both service options menus should show the same.

- When adding the same name and Number to the first four fields [Mobile, Home office, Extension number] to the phone book  a second time with  same information. When the entry is duplicated, the display will show "Duplicate Extrnsion No, Save Again?" NO soft key to go back or Yes or No.  After about 30 seconds the phone locks (5220) or resets (5330).

- The 5230 set does not correct remove deleted characters from the button name on the main display.