| o : | Doc. No. |  |
| --- | --- | --- |
| C.C. : | Country | ALL |
| From : M.W.Choi | Tel: E-mail: |  |
| Subject: Release OfficeServ 7200 LCP S/W V4.33 |  |  |

Overview :

Release LCP SW v4.33 – 140401

Contents :

- The SLI3 Long Loop check feature is now able to be turned off.

S/W Compatibility Table :

| MODEL | LCP | MP20 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| VERSION | V4.33 | V4.60~ |  |  |  |
| DATE | 14.04.01 | 12.02.06 |  |  |  |

Version Information :

| Board | Package Name | Version | Date | Description |
| --- | --- | --- | --- | --- |
| LCP | LPPSV433.PGM | V4.33 | 20140401 | SP S/W for LCP card |

Version History :

V4.33

- PKG : 2014.04.01

- Change Notes

1. The SLI3 Long Loop check feature is now able to be turned off.

: The SLI3 card has Long Loop Check and Disconnect feature for preventing overheat. But, in case of OS7200/OS7400, fans of the cabinet are enough to cool the card. So, we provide an option to turn off the feature. Now you can show the option in MMC861.

V4.32

- PKG : 2012.11.20

- Change Notes

1. SLI3 DTMF detection problem with a phone made by OPHIR TELECOM was fixed.

: Because of SLI3’s DC characteristic, the DTMF signal with a phone(made by OPHIR TELECOM) was distorted. We modified DC characteristic to support this phone.

V4.31

- PKG : 2012.04.27

- Change Notes

1. The SLI3 port lock-up problem when using message waiting feature in LCP cabinet was

fixed.

: For initialization of SLI3 cards, some parameters related to H/W characteristics have to be applied. But, LCP V4.30 skipped the procedure and SLI3 cards worked on default parameters of chip. We modified LCP to install appropriate parameters when initialization.

V4.30

- PKG : 2011.12.09

- Change Notes

1. SLI3 cards(16SLI3, 16SLI3s, 8SLI3, 8COMBO3) are now available.

: SLI3 cards(16SLI3, 16SLI3s, 8SLI3, 8COMBO3) were developed. We add service routine of SLI3 cards to the system.

2. SVMi-20i card is now available.

: SVMi-20i card was developed. We add the service routine of SVMi-20i card to the system.

3. LCP Upgrade(from V4.30 Alpha Test Version) Problem.

: If you already upgraded LCP to V4.30(Alpha Test Version), you may not be able to upgrade LCP anymore. V4.30(11.10.06 Alpha Test Version) has PGM upgrade problem. LCP need to be recovered by following step. We will send a requisite file(DPSME.H) with LCP PGM.

How to modify the LCP program via SIO. ** Download procedure Using RXTX.EXE 1. Copy the DPSME.H to the folder in which RXTX.exe is. 2. Connect the LCP card and PC with RS-232C cable and configure the com port (baud rate:19.2K) 3. Move into the ASCII mode. (F1) 4. After pressing the reset button, press the 'enter' key several times within 1~2 second. 5. Input the password 'dcsplussamsung'(small letter) 6. If you see 'SCP_boot>', enter the 'LO' 7. Enter 'Esc' key and then you will see the menu. Select 'Send file' and then input file name(DPSME.H). 8. Check the LED of LCP card whether it works well or not. After a long while, you can see the menu and then select the 'ASCII mode' 9. You can find some kind of information(checksum...). To change program, input 'RP'. 10. Flash change LED works again and at the end, you can find the restart message. then you may input 'GO' or press the reset button to restart the card. ** Example) Download using TERA TERM 1. Execute TERA TERM. 2. Connect the LCP card and PC with RS-232C cable and configure the com port (baud rate:19.2K) 3. Move into the ASCII mode. (F1) 4. After pressing the reset button, press the 'enter' key several times within 1~2 second. 5. Input the password 'dcsplussamsung'(small letter) 6. If you see 'SCP_boot>', enter the 'LO' Then you will see "Please start loading file (S-record) ! 7. In the manu bar, select 'Send file' and then input file name(DPSME.H). If there is binary checkbox below, you have to check it. 8. Check the LED of LCP card whether it works well or not. After a long while, you can see the menu and then select the 'ASCII mode' 9. You can find some kind of information(checksum...). To change program, input 'RP'. 10. Flash change LED works again and at the end, you can find the restart message. then you may input 'GO' or press the reset button to restart the card. If you use other SIO program, execute the same thing which act as 'send file' of rxtx. Regarding the remainder, the procedure is same with rxtx.

V4.20

-. PKG

. Main PKG : 04/03/2011

- Change Notes

1. The New OAS card(PCB V02) is available.

: New OAS card works in all slot of OS7200.

V4.19

-. PKG

. Main PKG : 30/08/2010

- Change Notes

1. The CNF24 card is available.

: The CNF24 card was developed. We add IPC feature between MP and CNF24.

V4.18

-. PKG

. Main PKG : 12/07/2010

- Change Notes

1. 16TRK/8TRK2 Trunk Port Lock up.

: There was a port lock up problem. The diagnostic feature was not worked on

16TRK/8TRK2. We fix the TRK diagnostic feature to work on 16TRK/8TRK2.

V4.17

-. PKG

. Main PKG : 29/04/2009

- Change Notes

1. MPS feature of OAS card is available.

: MPS feature of OAS card is required. We add MPS messages.

2. Changing inter digit time didn't be affected when using dial pulse feature.

: System counts inter digit time by 10ms. But inter digit time is set by 100ms. We modified

inter digit timer to count time by 100ms.

V4.16

-. PKG

. Main PKG : 12/05/2008

- Change Notes

1. Addition of the interface feature for OAS Card.

=> To use the Mobex feature, we need another mfr resource.

We added the interface feature for OAS Card. We need the interface feature the OAS Card work properly in Expansion cabinet(LCP)

2. LCP system restart in dial pulse feature.

=> We operated Dial pulse feature.  Because of memory fault, LCP S/W in OS7100 cabinet restarted. Memory fault in S/W was fixed.

3. We can't hear any tone in DGP.

=> When DLI card was initailized, S/W did not correctly assigned time-slot. We modified time-slot process.

4. DGP displayed only version.

=> If Timer job occurred during initial process, S/W did not correctly initialized DLI card. S/W initialize DLI card independently of Timer job.

5. DLI port changes to SLI port.

=> Digital phone send phone type when digital phone boot up. But the phone type is different often. We modify system to check the phone type 3 times.

V4.14

-. PKG

. Main PKG : 08/07/2008

- Change Notes

1. LCP is now available on OS7100 rack.

=> Using of LCP on OS7100 rack is required. We add rack type message and update channel mapping table.

2. CID feature is available in UK.

=> Using of CID in UK is required. CID feature in UK is added .

3. PRS is now working in SWEDEN and SPAIN.

=> PRS feature is modified to be worked in SWEDEN and SPAIN.

4. CRM board is available to OS7200

=> Old Engine(STL7065) will be replaced with New Engine(STC9604), and there's a necessity for using CRM board in OS7200. We modified the LCP software to adpot the CRM board to OS7200.

V4.12

-. PKG

. Main PKG : 11/09/2007

- Change Notes

1. New cards(8COMBO2, 8SLI2) are available.

=> We add the interface routine for two cards(8COMBO2, 8SLI2).

2. MW feature is available on 16SLI2/8SLI2/8COMBO2.

=> We add MW feature based on PRS.

3. PRS3 hangs up call on answer.

=> Base polarity value was stored before ringing state. But in PRS3 mode it is necessary the value to be stored after answer. So we modify system store the base polarity value after answer.

4. DGP with 8COMBO2 do not work sometimes.

=> When user power up system as soon as power down, QDASL of 8COMBO2 don't be initialized sometimes. We re-initialize 8COMBO2 when startup.

5. 16TRK/8TRK2 Initial problem

=> Some ports of 16TRK/8TRK2 did not work on specific cards combination. When 16TRK/8TRK2 is on initial phase, if trunk gain parameters are downloaded and system reset the slot, then initialization is failed. We modify TRK initial phase to block slot reset when LP40 download trunk gain parameters.

V4.11

-. PKG

. Main PKG : 08/01/2007

- Change Notes

1. When the country is set, the cid configuration is fixed.

=> We add the MMC to configure the standard of cid, receive level,

transmission level and the coding rule.

2. Four kind of new cards are developed.

=> We developed the new cards and add the interface routine for new cards.

3. Dial Pulse feature of TRK cards(16TRK and 8TRK2) is required.

=> We add Dial Pulse feature to 16TRK/8TRK2.

4. If user cancel MW when SLT's lamp is on, the lamp is not off.

=> We add the condition to blink control routine.

5. If TEPRIa card is E1/T1 mode, system can't display TEPRI version correctly.

=> We modify routine to process version information over 0x20.

6. CID receiver did not work within 5 sec after hook-on in New Zealand.

=> We add non-PRS feature to condition.

7. MW feature is available on 16SLI2/8SLI2/8COMBO2.

=> We add MW feature based on PRS.

V2.69

-. PKG

. Main PKG : 01/04/2007

-. Change Notes

. The SLI port of 8COMBO locks up when VMAA set is in MMC207

=> We changed the number of DLI port to start with eight

. Cross Talk on OS7200 when using Auto Answer Feature

=> At the end of playing the tone, we added inserting the conference group

number.

. Add MGI16 Interface

=> We add the interface software in LCP to operate the MGI16 card

. SDMF CID detection feature add

- We modified the LCP software to detect the SDMF format CID

V2.63

-. PKG

. Main PKG : 02/15/2006

-. Change Notes

. The variable of RCM2 initial routine was modified to be initialized properly.

. Upgrade DLI Recovery Logic to reinitial and check initial status

. Modified the DSP program to wait and detect the seizure signal instead of CID header first

. Modified We modified the CID processing routine of DDN parameter

. Add to TEPRI MSG Transformation Index related to H/W information

. Add MGI16 interface service lootine to OS100/500/7200.